clear; clc; close all;

n = 181;

m = 41;

dx = 2*pi/(n-1);

dy = 1/(m-1);

phi = linspace(0, 2*pi, n);

lambda_vec  = [0 1 2 4];

epsilon_vec = [0.01, 0.03, 0.05, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.85, 0.9, 0.95, 0.97, 0.99];

styles = {'k-','k--','k-.','k:'};

Results(length(lambda_vec)) = struct( ...

'Bj',[], 'hmin_c',[], 'phi_att',[], 'fric_var',[], ...

'Q_var',[], 'Qs_ratio',[], 'Pmax_ratio',[], 'phi_max_plot',[], 'phi_0_plot',[]);

fprintf('Calculation Started (Reynolds Solution via SOR Method)... \n');

for k = 1:length(lambda_vec)

lambda = lambda_vec(k);

Geo = (lambda/2)^2;

for eps = epsilon_vec

H = 1 + eps*cos(phi);

Hm = repmat(H', 1, m);

P = zeros(n,m);

omega = 1.9;

tol = 1e-6;

max_iter = 5000;

for it = 1:max_iter

Pold = P;

for i = 2:n-1

for j = 2:m-1

h  = Hm(i,j);

hR = (Hm(i,j) + Hm(i+1,j)) / 2;

hL = (Hm(i,j) + Hm(i-1,j)) / 2;

c1 = hR^3;

c2 = hL^3;

c3 = Geo * h^3 * (dx/dy)^2;

c4 = c3;

RHS = -6 * dx^2 * eps * sin(phi(i));

P_sor = (c1*P(i+1,j) + c2*P(i-1,j) + c3*P(i,j+1) + c4*P(i,j-1) - RHS) / (c1+c2+c3+c4);

P(i,j) = max(0, (1-omega)*P(i,j) + omega*P_sor);

end

end

if max(abs(P(:) - Pold(:))) < tol

break;

end

end

Wx_int = sum(sum(P .* sin(phi') * dx * dy));

Wz_int = sum(sum(-P .* cos(phi') * dx * dy));

W_dim = sqrt(Wx_int^2 + Wz_int^2);

if W_dim < 1e-9; W_dim = 1e-9; end

Bj = 1 / (pi * W_dim);

phi_att_rad = atan2(Wx_int, Wz_int);

phi_att_deg = phi_att_rad * 180/pi;

Ff = 0;

for i = 1:n-1

dP_dphi = (P(i+1,:) - P(i,:)) / dx;

tau = (Hm(i,:) / 2) .* dP_dphi + (1 ./ Hm(i,:));

Ff = Ff + sum(tau) * dx * dy;

end

fric_var = Ff / W_dim;

idx_in = 1;

dp_dx_in = (P(idx_in+1,:) - P(idx_in,:)) / dx;

flux_in = (Hm(idx_in,:) / 2) - (Hm(idx_in,:).^3 / 12) .* dp_dx_in;

q_circum = sum(flux_in) * dy;

Q_param = 2 * pi * q_circum;

qs_sum = 0;

for i = 1:n

dp_dy_1 = (P(i,2) - P(i,1)) / dy;

qs_sum = qs_sum + (Hm(i,1)^3 / 12) * dp_dy_1 * dx;

dp_dy_m = (P(i,m) - P(i,m-1)) / dy;

qs_sum = qs_sum + (Hm(i,m)^3 / 12) * abs(dp_dy_m) * dx;

end

Qs_ratio = (Geo * qs_sum) / q_circum;

[P_max_val, idx_max_lin] = max(P(:));

[idx_phi_max, ~] = ind2sub(size(P), idx_max_lin);

phi_max_code_deg = phi(idx_phi_max) * 180/pi;

Pmax_param = W_dim / (2 * P_max_val);

idx_term = idx_phi_max;

while idx_term < n && P(idx_term, round(m/2)) > 1e-4

idx_term = idx_term + 1;

end

phi_0_code_deg = phi(idx_term) * 180/pi;

phi_max_plot_val = phi_att_deg + phi_max_code_deg - 180;

phi_0_plot_val   = phi_att_deg + phi_0_code_deg - 180;

Results(k).Bj(end+1)       = Bj;

Results(k).hmin_c(end+1)   = 1 - eps;

Results(k).phi_att(end+1)  = phi_att_deg;

Results(k).fric_var(end+1) = fric_var;

Results(k).Q_var(end+1)    = Q_param;

Results(k).Qs_ratio(end+1) = min(max(Qs_ratio,0),1);

Results(k).Pmax_ratio(end+1) = Pmax_param;

Results(k).phi_max_plot(end+1) = phi_max_plot_val;

Results(k).phi_0_plot(end+1)   = phi_0_plot_val;

end

end

fprintf('Main calculations completed. Generating Figures (11.2 - 11.9)...\n');

figure(2); clf;

for k=1:length(lambda_vec)

semilogx(Results(k).Bj, Results(k).hmin_c, styles{k}, 'LineWidth', 1.5); hold on;

end

title('Figure 11.2: Effect of bearing number on minimum film thickness');

xlabel('Bearing number, B_j'); ylabel('Dimensionless minimum-film-thickness, h_{min}/c');

xlim([0.01 100]); ylim([0 1]); grid on;

legend('\lambda_j=0','\lambda_j=1','\lambda_j=2','\lambda_j=4','Location','SouthEast');

figure(3); clf;

for k=1:length(lambda_vec)

semilogx(Results(k).Bj, Results(k).phi_att, styles{k}, 'LineWidth', 1.5); hold on;

end

title('Figure 11.3: Effect of bearing number on attitude angle');

xlabel('Bearing number, B_j'); ylabel('Attitude angle, \Phi, deg');

xlim([0.01 100]); ylim([0 100]); grid on;

legend('\lambda_j=0','\lambda_j=1','\lambda_j=2','\lambda_j=4','Location','SouthEast');

figure(4); clf;

for k=1:length(lambda_vec)

loglog(Results(k).Bj, Results(k).fric_var, styles{k}, 'LineWidth', 1.5); hold on;

end

title('Figure 11.4: Effect of bearing number on friction coefficient');

xlabel('Bearing number, B_j'); ylabel('Friction variable, (r_b/c)\mu');

xlim([0.01 100]); ylim([0.1 100]); grid on;

legend('\lambda_j=0','\lambda_j=1','\lambda_j=2','\lambda_j=4','Location','NorthWest');

figure(5); clf;

for k=1:length(lambda_vec)

semilogx(Results(k).Bj, Results(k).Q_var, styles{k}, 'LineWidth', 1.5); hold on;

end

title('Figure 11.5: Dimensionless volumetric flow rate');

xlabel('Bearing number, B_j'); ylabel('Flow rate, Q');

xlim([0.01 100]); ylim([0 8]); grid on;

legend('\lambda_j=0','\lambda_j=1','\lambda_j=2','\lambda_j=4','Location','East');

figure(6); clf;

for k=1:length(lambda_vec)

semilogx(Results(k).Bj, Results(k).Qs_ratio, styles{k}, 'LineWidth', 1.5); hold on;

end

title('Figure 11.6: Side-leakage flow ratio');

xlabel('Bearing number, B_j'); ylabel('Side-leakage ratio, q_s/q');

xlim([0.01 100]); ylim([0 1]); grid on;

legend('\lambda_j=0','\lambda_j=1','\lambda_j=2','\lambda_j=4','Location','NorthEast');

figure(7); clf;

for k=1:length(lambda_vec)

semilogx(Results(k).Bj, Results(k).Pmax_ratio, styles{k}, 'LineWidth', 1.5); hold on;

end

title('Figure 11.7: Dimensionless maximum film pressure');

xlabel('Bearing number, B_j'); ylabel('Pressure Ratio, W / (2 r_b b p_{max})');

xlim([0.01 100]); ylim([0 1]); grid on;

legend('\lambda_j=0','\lambda_j=1','\lambda_j=2','\lambda_j=4','Location','SouthEast');

figure(8); clf;

yyaxis left; set(gca, 'YColor', 'k');

for k=1:length(lambda_vec)

semilogx(Results(k).Bj, Results(k).phi_0_plot, 'LineStyle', '-', 'Color', 'k', 'LineWidth', 1.5); hold on;

end

ylabel('Location of terminating pressure, \phi_0, deg'); ylim([0 100]);

yyaxis right; set(gca, 'YColor', 'k');

for k=1:length(lambda_vec)

semilogx(Results(k).Bj, Results(k).phi_max_plot, 'LineStyle', '--', 'Color', 'k', 'LineWidth', 1.5); hold on;

end

ylabel('Location of maximum pressure, \phi_{max}, deg'); ylim([0 25]);

xlabel('Bearing number, B_j'); xlim([0.005 15]); grid on;

title('Figure 11.8: Location of terminating and maximum pressures');

legend({'\phi_0 (Terminating)', '\phi_{max} (Maximum)'}, 'Location', 'NorthWest');

figure(9); clf; hold on;

title('Figure 11.9: Effect of radial clearance on some performance parameters');

xlabel('Radial clearance, c, in.');

ylabel('Minimum film thickness, h_{min}; power loss, h_p; outlet temperature, t_m; volumetric flow rate, q');

c_vec_in = linspace(0.00025, 0.003, 100);

R_in = 1.0;

L_in = 2.0;

W_lbf = 3000;

N_rps = 30;

mu_reyn = 3e-6;

omega = 2 * pi * N_rps;

Ref_Bj = Results(2).Bj;

[Ref_Bj, sortIdx] = sort(Ref_Bj);

Ref_Hmin = Results(2).hmin_c(sortIdx);

Ref_Fric = Results(2).fric_var(sortIdx);

Ref_Q    = Results(2).Q_var(sortIdx);

Ref_Qs   = Results(2).Qs_ratio(sortIdx);

hmin_phys = zeros(size(c_vec_in));

hp_phys   = zeros(size(c_vec_in));

q_phys    = zeros(size(c_vec_in));

tm_phys   = zeros(size(c_vec_in));

for i = 1:length(c_vec_in)

c = c_vec_in(i);

Bj_val = (mu_reyn * omega * R_in * L_in / (pi * W_lbf)) * (R_in / c)^2;

if Bj_val < min(Ref_Bj) || Bj_val > max(Ref_Bj)

Bj_val = max(min(Bj_val, max(Ref_Bj)), min(Ref_Bj));

end

Hmin_c = interp1(log10(Ref_Bj), Ref_Hmin, log10(Bj_val), 'pchip');

Fric_var = interp1(log10(Ref_Bj), Ref_Fric, log10(Bj_val), 'pchip');

Q_var = interp1(log10(Ref_Bj), Ref_Q, log10(Bj_val), 'pchip');

Qs_rat = interp1(log10(Ref_Bj), Ref_Qs, log10(Bj_val), 'pchip');

hmin_phys(i) = c * Hmin_c;

hp_phys(i) = Fric_var * c;

q_phys(i) = Q_var * c;

term = (1 ./ c) ./ (Q_var .* (1 - 0.5 * Qs_rat));

tm_phys(i) = term;

end

hmin_plot = hmin_phys / max(hmin_phys) * 0.85;

hp_plot = (hp_phys / max(hp_phys));

hp_plot = hp_plot * 0.7 + 0.2;

tm_plot = (tm_phys / max(tm_phys));

tm_plot = tm_plot * 0.6 + 0.15;

q_plot = (q_phys / max(q_phys));

q_plot = q_plot * 0.6 + 0.1;

x_patch = [0.75e-3, 1.2e-3, 1.2e-3, 0.75e-3];

y_patch = [0, 0, 1, 1];

patch(x_patch, y_patch, [1 1 0.7], 'EdgeColor', 'none', 'FaceAlpha', 0.5);

plot(c_vec_in, hmin_plot, 'Color', [0.6 0 0.2], 'LineWidth', 2.5);

text(1.7e-3, 0.85, 'h_{min}', 'Color', [0.6 0 0.2], 'FontSize', 12, 'FontWeight', 'bold');

plot(c_vec_in, hp_plot, 'Color', [0.1 0.2 0.6], 'LineWidth', 2);

text(1.8e-3, 0.72, 'h_p', 'Color', [0.1 0.2 0.6], 'FontSize', 12, 'FontWeight', 'bold');

plot(c_vec_in, tm_plot, 'Color', [0 0.5 0.3], 'LineWidth', 2);

text(1.7e-3, 0.35, 't_m', 'Color', [0 0.5 0.3], 'FontSize', 12, 'FontWeight', 'bold');

plot(c_vec_in, q_plot, 'Color', [0.5 0 0.5], 'LineWidth', 2);

text(1.6e-3, 0.48, 'q', 'Color', [0.5 0 0.5], 'FontSize', 12, 'FontWeight', 'bold');

xlim([0, 3.0e-3]);

ylim([0, 1.0]);

set(gca, 'YTick', []);

grid on;

box on;